3GPP TSG SA WG3-LI						S3LI02_071

9-11 April 2002

Orlando, Florida, USA

Source:		Manfred  Beierer, Reg TP

Contact:		manfred.beierer@regtp.de

##### Title:			ASN.1 – Imports and Exports of Parameters

Document for:	Discussion/Approval

# Introduction

At the at the ASN.1 workshop at ETSI, 19.03.2002, amongst others the following recommendations were made:

- Use extensibility operator in code - Use specific IMPORTS - Use OID tree and identify location in the tree in the header of the ASN.1 module - In every module ensure EXPORTS ALL its parameters - Don't redesign parameters/types which already exist!!

As currently there is considerable overlap in the parameters definition between TS 33.108 and ES 201 671, it is proposed to apply recommendations 2 and 5  and to import parameters already designed in ES 201 671, taking into account that for technology specific parameters SA3  LI should have the primary responsibility. This contribution, as a starting point for discussion, makes a proposal which parameter should be imported.

# Candidate Parameters for Import

The liaison statement from SEC LI to SA3 LI contains a list with candidate parameters for import. This list is used for additional comments and recommendations how to proceed for every parameter. It should be noted that SEC LI at its recent meeting has accepted a Change Request in order to offer all the parameters of ES 201 671 for export (see embedded CR16).

| Parameter in 33.108 | Difference between TS 33.108 and ES 201 671 | Proposed primary responsibility | ###### Actions required for SA3 LI |
| --- | --- | --- | --- |
| Network-Identifier | No difference | SEC LI | to be imported |
| Network-Element-Identifier | IP-Address new, to be added to ES 201671 | SEC LI | to be imported, CR for ES to add ' iP-Address' |
| TimeStamp | No difference | SEC LI | already imported |
| PartyInformation | e164-Format new, to be added to ES 201671 | SEC LI | to be imported, CR for ES to add e164-Format and to make originating, terminating and forwarde-to party optional. |
| Location | No difference | SEC LI | to be imported |
| GSMLocation | No difference | SEC LI | to be imported |
| UMTSLocation | No difference | SA3 LI | inform SEC LI to import this parameter |
| GeographicalCoordinates | No difference | SEC LI | to be imported |
| GA-Point | No difference | SEC LI | to be imported |
| GA-PointWithUnCertainty | No difference | SEC LI | to be imported |
| maxNrOfPoints | No difference | SEC LI | to be imported |
| SMS-report | 2 values not needed in UMTS, | SA3 LI | add communicationIdentifier and timeStamp, make them optional and inform SEC LI to import this parameter |
| LawfulInterceptionIdentifier | No difference | SEC LI | already imported |
| National-Parameters | No difference | SEC LI | to be imported |
| GPRSCorrelationNumber | The size was changed, to be aligned in the ES 201671 | SA3 LI | inform SEC LI to import this parameter |
| GPRSEvent | the new value (13) should be added to the ES 201671 | SA3 LI | inform SEC LI to import this parameter |
| Services-Data-Information | no difference | SEC LI ? | inform SEC LI to import this parameter |
| GPRS-parameters | no difference | SA3 LI | inform SEC LI to import this parameter |
| GPRSOperationErrorCode | no difference | SA3 LI | inform SEC LI to import this parameter |
| DataNodeAddress | no difference | SEC LI | to be imported |
| IPAddres | IP-assignment new, to be added to ES 201671 | SEC LI | to be imported, CR for ES to add IP-assignment |
| IP-value | no difference | SEC LI | to be imported |
| X25Address | no difference | SEC LI | to be imported |
| Qos | new, to be added to the ES 201671 | SEC LI ? | to be imported, CR for ES to add the UMTS-values or rename QOS to QOS-UMTS |

# Conclusion

As a result of the  proposed import and exports of the parameters several advantages are achieved:

- The parameters are defined in the technical body where the expertise is resided,

- No overlap and misalignments between the parameters used in the individual specifications,

- Reduced complexity for the implementations in MDs and LEMFs serving not only UMTS but also other technologies.

The proposed changes do not result in a change of the ASN.1 code for the existing specifications. However, care must be taken for backward and overall compatibility. Those who have the primary responsibility for given parameters should be ready to accept proposals for additions from other technologies, whereby these additional parameters or values could be classified as optional if not needed everywhere.