**** VCom v0.2b ****

(1 port inbound telnetd)

NOTE: Because this program will be shareware or maybe commercialware,

it supports a maximum of 40 connections (not simultaneously).  After

40 users have telnetted to your site, you must reboot to reset the

counter.

Version History

---------------

0.1b    Initial release.

0.2b    Revised release, although still in beta development.

Enhancements:

Speed improvement, both to connection and the system (win95) while vcom is running

Fixed bug with 16550 detection

Server starts enabled

A message box informing the user that vcom.vxd has not been loaded properly

if this is the case; in the last version, vcom.386 not being loaded properly

resulted in a page fault.

Installation

------------

1. Add a device statement in your system.ini for vcom.vxd:

[386Enh]

device=x:\vcom.vxd

where x is the path to vcom.vxd

2. If you want a base address other than 2a0h and/or irq other than 7, then

specify the base address and irq for the virtual com port in your

system.ini:

[vcom] <--- NOTE: This has changed from the last release.

port=2a0h  <--- port and irq setting must be in hex and terminate with a "h"

irq=7h

If your system.ini does not contain a [vcom] section, the port will default

to 2a0h and the irq will default to 7.

NOTE: In this version no error-checking is implemented on port/irq selection,

so choose a port and irq that is not in use by another device.  You

can find valid candidates by right-clicking on "My Computer," selecting

properties, selecting the "Device Manager tab," and double-clicking on

"Computer."  In future multi-port versions, all ports will share a

single irq.

3. Reboot your computer.  After doing so, launch vcom.exe and then the DOS

application that you would like to use with the virtual com port.

4. Test your program by telnetting to 127.0.0.1.

Virtual Modem Commands

----------------------

All AT commands other than "ATA" return "OK," and "ATA" returns "OK" if a

ring has been detected or "NO CARRIER" otherwise.  "RING" and "CONNECT ..."

messages are also returned by the virtual modem.  "ATH0" and "+++" are not

supported, so the remote party must disconnect, which will return a "NO

CARRIER" message.  In this version DTR checking is not implemented, so the

only ways to hang up are: 1) the remote user disconnects or 2) disable then

enable the server.  The virtual modem is one of the areas of vcom that still

needs a significant amount of work.

Bugs

----

I am aware of the few rare page faults, but I have not had much time to test

this version, so expect bugs.

Future Enhancements

-------------------

Significant performance improvement

Multiple ports (using 1 irq for all the ports)

Outbound connection via "ATDT <ip address>"

Improved virtual modem emulation including ATH0 command

Server as system tray icon/control panel applet

Caller ID (?)

Contact Information

-------------------

The VCom support BBS should be up 24/7 at vcom.gt.ed.net.

The web page url is http://www.netime.com/~infrared/vcom.

I can be reached at infrared@netime.com or root@vcom.gt.ed.net.